CT861 Assignment 1 Report

Name: Cian Mac Liatháin

ID: 11476078

Problem 1.1

The Memory Address Register (MAR) is a hidden register found on the CPU that is connected to the address bus and used to store the memory address of the next instruction to be executed by the CPU.

The Memory Buffer Register (MBR, also known as Memory Data Register (MDR)) is a two-way hidden register that is used to store data that was either read from the MAR or will be written immediately to the MAR.

The use of these registers allows the CPU more freedom to read and work with data cached in the registers without having to re-request it from the memory location.

Example:

The Opcode for Add X, add the value stored at address X into the Accumulator, in MARIE is 3 in hexadecimal or 0011 in binary. An instruction of 001100010100 then means to add the value found at address 00010100 to the accumulator:

MAR <- 00010100 // The memory address is loaded into the MAR

MBR <- M[MAR] // the memory address is accessed, and the value stored there is copied into MBR

AC <- AC + MBR // this value is added to the accumulator

Problem 1.2

MARIE supports the two different addressing mode of:

With Direct Memory Addressing the address field, the second part of the instruction code after the opcode, is the actual memory address of the operand to be used as part of the instruction. In contrast, with Indirect Memory Addressing this address field is in fact an effective address of the operand, meaning it is a reference to a further address location at which the operand is to be found to be used.

Here is an example showing the difference between Load X (Direct Memory Addressing) and LoadI X (Indirect Memory Addressing):

Load X

MAR <- X // The address is loaded into MAR

MBR <- M[MAR] // The value stored at address MAR is stored in MBR

AC <- MBR //This value is then loaded into the accumulator, as it is directly the intended operand

LoadI X

MAR <- X //Loads the address X into MAR

MBR <- M[MAR] // Loads the value found at MAR into MBR, this value is a further address whose 			value stored at that second address is the operand

MAR <- M[MBR] // MAR is loaded with the indirect address value, effectively now continuing in the 			same way as the Direct Memory Addressing above

MBR <- M[MAR]

AC <- MBR //Accumulator is loaded with the intended operand