CT861 Assignment 2 - WireShark Report

Name: Cian Mac Liatháin

ID: 1146078

Part 1: Ping

1.1. Print Packets

- 2.613290       192.168.23.105        104.18.142.17         ICMP     98     Echo (ping)

request  id=0xed0e, seq=0/0, ttl=64 (reply in 4)

- 2.677405       104.18.142.17         192.168.23.105        ICMP     98     Echo (ping)

reply    id=0xed0e, seq=0/0, ttl=57 (request in 3)

- 3.618418       192.168.23.105        104.18.142.17         ICMP     98     Echo (ping)

request  id=0xed0e, seq=1/256, ttl=64 (reply in 7)

- 3.664570       104.18.142.17         192.168.23.105        ICMP     98     Echo (ping)

reply    id=0xed0e, seq=1/256, ttl=57 (request in 6)

- 4.618639       192.168.23.105        104.18.142.17         ICMP     98     Echo (ping)

request  id=0xed0e, seq=2/512, ttl=64 (reply in 19)

- 4.665369       104.18.142.17         192.168.23.105        ICMP     98     Echo (ping)

reply    id=0xed0e, seq=2/512, ttl=57 (request in 18)

1.2. Identify Protocols

Network protocol: Internet Communication Message Protocol (ICMP)

Data Link protocol: Ethernet II

The network layer Internet Protocol v4 destination address is a 32-bit dotted logical address that allows for communication between networks by uniquely identifying network-enabled devices. A combination of public and private IP addresses helps traffic packets through the internet in order to arrive at the correct Local Area Network.  IP can change and be re-assigned, meaning that the same computer could be identified by different IP addresses but only one at a time. The data link layer Media Access Control address is a unique, static, 48-bit dashed hexadecimal physical address assigned to any network interface card upon manufacture and used within Local Area Networks to route packets to their final destination.

Part 2: Web Server

2.1. Print Packets

38 total packets, 12 shown

4 2.212491       192.168.23.105        13.248.207.191        TCP      78     51544 → 80

[SYN, ECN, CWR] Seq=0 Win=65535 Len=0 MSS=1460 WS=64 TSval=558733140 TSecr=0 SACK_PERM=1

Frame 4: 78 bytes on wire (624 bits), 78 bytes captured (624 bits) on interface en0, id 0 Ethernet II, Src: Apple_82:58:36 (3c:22:fb:82:58:36), Dst: 4a:ad:3a:02:78:6d (4a:ad:3a:

02:78:6d)

Internet Protocol Version 4, Src: 192.168.23.105, Dst: 13.248.207.191

Transmission Control Protocol, Src Port: 51544, Dst Port: 80, Seq: 0, Len: 0

6 2.466760       192.168.23.105        13.248.207.191        TCP      78     51545 → 80

[SYN, ECN, CWR] Seq=0 Win=65535 Len=0 MSS=1460 WS=64 TSval=558733393 TSecr=0 SACK_PERM=1

Frame 6: 78 bytes on wire (624 bits), 78 bytes captured (624 bits) on interface en0, id 0 Ethernet II, Src: Apple_82:58:36 (3c:22:fb:82:58:36), Dst: 4a:ad:3a:02:78:6d (4a:ad:3a:

02:78:6d)

Internet Protocol Version 4, Src: 192.168.23.105, Dst: 13.248.207.191

Transmission Control Protocol, Src Port: 51545, Dst Port: 80, Seq: 0, Len: 0

9 3.172810       13.248.207.191        192.168.23.105        TCP      74     80 → 51544

[SYN, ACK, ECN] Seq=0 Ack=1 Win=65535 Len=0 MSS=1420 SACK_PERM=1 TSval=2537925599 TSecr=558733140 WS=256

Frame 9: 74 bytes on wire (592 bits), 74 bytes captured (592 bits) on interface en0, id 0 Ethernet II, Src: 4a:ad:3a:02:78:6d (4a:ad:3a:02:78:6d), Dst: Apple_82:58:36 (3c:22:fb:

82:58:36)

Internet Protocol Version 4, Src: 13.248.207.191, Dst: 192.168.23.105

Transmission Control Protocol, Src Port: 80, Dst Port: 51544, Seq: 0, Ack: 1, Len: 0

11 3.172814       13.248.207.191        192.168.23.105        TCP      74     80 → 51545

[SYN, ACK, ECN] Seq=0 Ack=1 Win=65535 Len=0 MSS=1420 SACK_PERM=1 TSval=2537017737 TSecr=558733393 WS=256

Frame 11: 74 bytes on wire (592 bits), 74 bytes captured (592 bits) on interface en0, id 0 Ethernet II, Src: 4a:ad:3a:02:78:6d (4a:ad:3a:02:78:6d), Dst: Apple_82:58:36 (3c:22:fb:

82:58:36)

Internet Protocol Version 4, Src: 13.248.207.191, Dst: 192.168.23.105

Transmission Control Protocol, Src Port: 80, Dst Port: 51545, Seq: 0, Ack: 1, Len: 0

19 3.172982       192.168.23.105        13.248.207.191        TCP      66     51544 → 80

[ACK] Seq=1 Ack=1 Win=132352 Len=0 TSval=558734095 TSecr=2537925599

Frame 19: 66 bytes on wire (528 bits), 66 bytes captured (528 bits) on interface en0, id 0 Ethernet II, Src: Apple_82:58:36 (3c:22:fb:82:58:36), Dst: 4a:ad:3a:02:78:6d (4a:ad:3a:

02:78:6d)

Internet Protocol Version 4, Src: 192.168.23.105, Dst: 13.248.207.191

Transmission Control Protocol, Src Port: 51544, Dst Port: 80, Seq: 1, Ack: 1, Len: 0

20 3.173058       192.168.23.105        13.248.207.191        TCP      66     51545 → 80

[ACK] Seq=1 Ack=1 Win=132352 Len=0 TSval=558734095 TSecr=2537017737

Frame 20: 66 bytes on wire (528 bits), 66 bytes captured (528 bits) on interface en0, id 0 Ethernet II, Src: Apple_82:58:36 (3c:22:fb:82:58:36), Dst: 4a:ad:3a:02:78:6d (4a:ad:3a:

02:78:6d)

Internet Protocol Version 4, Src: 192.168.23.105, Dst: 13.248.207.191

Transmission Control Protocol, Src Port: 51545, Dst Port: 80, Seq: 1, Ack: 1, Len: 0

24 3.173327       192.168.23.105        13.248.207.191        HTTP     691    GET /

siteSettings.js HTTP/1.1

Frame 24: 691 bytes on wire (5528 bits), 691 bytes captured (5528 bits) on interface en0, id 0 Ethernet II, Src: Apple_82:58:36 (3c:22:fb:82:58:36), Dst: 4a:ad:3a:02:78:6d (4a:ad:3a:

02:78:6d)

Internet Protocol Version 4, Src: 192.168.23.105, Dst: 13.248.207.191

Transmission Control Protocol, Src Port: 51544, Dst Port: 80, Seq: 1, Ack: 1, Len: 625 Hypertext Transfer Protocol

29 3.239754       13.248.207.191        192.168.23.105        TCP      66     80 → 51544

[ACK] Seq=1 Ack=626 Win=251904 Len=0 TSval=2537925699 TSecr=558734095

Frame 29: 66 bytes on wire (528 bits), 66 bytes captured (528 bits) on interface en0, id 0 Ethernet II, Src: 4a:ad:3a:02:78:6d (4a:ad:3a:02:78:6d), Dst: Apple_82:58:36 (3c:22:fb:

82:58:36)

Internet Protocol Version 4, Src: 13.248.207.191, Dst: 192.168.23.105

Transmission Control Protocol, Src Port: 80, Dst Port: 51544, Seq: 1, Ack: 626, Len: 0

30 3.259459       13.248.207.191        192.168.23.105        HTTP     999    HTTP/1.1 404Not Found  (text/html)

Frame 30: 999 bytes on wire (7992 bits), 999 bytes captured (7992 bits) on interface en0, id 0 Ethernet II, Src: 4a:ad:3a:02:78:6d (4a:ad:3a:02:78:6d), Dst: Apple_82:58:36 (3c:22:fb: 82:58:36)

/var/folders/w8/_v0xtss15_j5rq6cr4xsjdzm0000gn/T//wireshark_Wi-Fi_20220131154939_xIvRtG.pcapng 38 total packets, 12 shown

Internet Protocol Version 4, Src: 13.248.207.191, Dst: 192.168.23.105

Transmission Control Protocol, Src Port: 80, Dst Port: 51544, Seq: 1, Ack: 626, Len: 933 Hypertext Transfer Protocol

Line-based text data: text/html (9 lines)

32 3.259534       192.168.23.105        13.248.207.191        TCP      66     51544 → 80

[ACK] Seq=626 Ack=934 Win=131392 Len=0 TSval=558734180 TSecr=2537925699

Frame 32: 66 bytes on wire (528 bits), 66 bytes captured (528 bits) on interface en0, id 0 Ethernet II, Src: Apple_82:58:36 (3c:22:fb:82:58:36), Dst: 4a:ad:3a:02:78:6d (4a:ad:3a:

02:78:6d)

Internet Protocol Version 4, Src: 192.168.23.105, Dst: 13.248.207.191

Transmission Control Protocol, Src Port: 51544, Dst Port: 80, Seq: 626, Ack: 934, Len: 0

34 4.197127       13.248.207.191        192.168.23.105        TCP      74     [TCP

Retransmission] 80 → 51545 [SYN, ACK, ECN] Seq=0 Ack=1 Win=65535 Len=0 MSS=1420 SACK_PERM=1 TSval=2537017841 TSecr=558733393 WS=256

Frame 34: 74 bytes on wire (592 bits), 74 bytes captured (592 bits) on interface en0, id 0 Ethernet II, Src: 4a:ad:3a:02:78:6d (4a:ad:3a:02:78:6d), Dst: Apple_82:58:36 (3c:22:fb:

82:58:36)

Internet Protocol Version 4, Src: 13.248.207.191, Dst: 192.168.23.105

Transmission Control Protocol, Src Port: 80, Dst Port: 51545, Seq: 0, Ack: 1, Len: 0

35 4.197191       192.168.23.105        13.248.207.191        TCP      66     [TCP Dup ACK

20#1] 51545 → 80 [ACK] Seq=1 Ack=1 Win=132352 Len=0 TSval=558735116 TSecr=2537017737

Frame 35: 66 bytes on wire (528 bits), 66 bytes captured (528 bits) on interface en0, id 0 Ethernet II, Src: Apple_82:58:36 (3c:22:fb:82:58:36), Dst: 4a:ad:3a:02:78:6d (4a:ad:3a:

02:78:6d)

Internet Protocol Version 4, Src: 192.168.23.105, Dst: 13.248.207.191

Transmission Control Protocol, Src Port: 51545, Dst Port: 80, Seq: 1, Ack: 1, Len: 0

2.2. Identify Protocols found in filtered capture1

Application: HyperText Transfer Protocol

Transport: Transport Control Protocol

Network: Internet Protocol v4

Data Link: Ethernet II

2.3. Print TCP and IP Headers of a packet. Selected packet 24

Internet Protocol Version 4, Src: 192.168.23.105, Dst: 13.248.207.191

0100 .... = Version: 4
	.... 0101 = Header Length: 20 bytes (5)
	Differentiated Services Field: 0x02 (DSCP: CS0, ECN: ECT(0))

0000 00.. = Differentiated Services Codepoint: Default (0)
		.... ..10 = Explicit Congestion Notification: ECN-Capable 	Transport codepoint '10' (2)

Total Length: 677

Identification: 0x0000 (0)

Flags: 0x4000, Don't fragment

0... .... .... .... = Reserved bit: Not set

.1.. .... .... .... = Don't fragment: Set

..0. .... .... .... = More fragments: Not set 	 		Fragment offset: 0								Time to live: 64									Protocol: TCP (6)									Header checksum: 0x8288 [validation disabled]				[Header checksum status: Unverified]

Source: 192.168.23.105

Destination: 13.248.207.191

The functionality of the IP header is to identify the source and destination of the packet as well as routing rules to ensure it arrives as intended. The IP header of this packet has a length of 20 bytes. For this GET request, the source IPv4 address is my laptop 192.168.23.105 and the destination address of 13.248.207.191 is the IP address of www.acadamh.ie. The 'Don't fragment' flag is set, indicating routers that this packet is not to be fragmented. The time to live of 64 indicates the limit to the number of hops the packet can take between the source and destination. The protocol number of 6 identifies TCP as the type of transport packet. The packet checksum can be used to confirm the validity of the packet and its data payload.

Transmission Control Protocol, Src Port: 51544, Dst Port: 80, Seq: 1, Ack: 1, Len: 625

Source Port: 51544

Destination Port: 80

[Stream index: 0]									[TCP Segment Len:	625]								Sequence number: 1    (relative sequence number)				Sequence number (raw): 2179067324

[Next sequence number: 626 (relative sequence number)] 		Acknowledgment number: 1 (relative ack number) 				Acknowledgment number (raw): 3342891478					1000 .... = Header Length: 32 bytes (8)

Flags: 0x018 (PSH, ACK)
		000. .... .... = Reserved: Not set
		...0 .... .... = Nonce: Not set
		.... 0... .... = Congestion Window Reduced (CWR): Not set 			.... .0.. .... = ECN-Echo: Not set
		.... ..0. .... = Urgent: Not set
		.... ...1 .... = Acknowledgment: Set
		.... .... 1... = Push: Set
		.... .... .0.. = Reset: Not set
		.... .... ..0. = Syn: Not set

.... .... ...0 = Fin: Not set
		[TCP Flags: ·······AP···]

Window size value: 2068
	[Calculated window size: 132352]
	[Window size scaling factor: 64]
	Checksum: 0xf6e1 [unverified]
	[Checksum Status: Unverified]
	Urgent pointer: 0
	Options: (12 bytes), No-Operation (NOP), No-Operation (NOP), Timestamps

TCP Option - No-Operation (NOP)

Kind: No-Operation (1)

TCP Option - No-Operation (NOP)

Kind: No-Operation (1)

TCP Option - Timestamps: TSval 558734095, TSecr 2537925599 				Kind: Time 	Stamp Option (8)
		Length: 10
		Timestamp value: 558734095

Timestamp echo reply: 2537925599

[SEQ/ACK analysis]								[iRTT: 0.960491000 seconds]
	[Bytes in flight: 625]
	[Bytes sent since last PSH flag: 625]

[Timestamps]
	[Time since first frame in this TCP stream: 0.960836000 seconds] 		[Time since previous frame in this TCP stream: 0.000345000 seconds]

TCP payload (625 bytes)

TCP header contains data required to establish and maintain a TCP connection. To identify the type of packet and indicate what stage the TCP connection is currently in. Packets flagged as urgent (URG) should be processed immediately, acknowledge (ACK) confirm that previously sent packages have been receved, push (PSH) are passed on directly to the requesting application, reset (RST) signifies the connection is broken, synchronise (SYN) initiates a connection or finish (FIN) closes a connection. This packet has the ACK and PSH bits set.

The TCP header has a length of 32 bytes, 32 bits of this is the sequence number that helps keep packets ordered by identifying their place in the stream. This packet is identified with sequence number 2179067324, or the relative sequence number of 1 (which can be easier to use for comparison by identifying the packet in the context of this segment of the TCP conversation). Additionally, its acknowledgement number of 3342891478 identifies relatively as 1 acknowledgement packet in this segment of the TCP stream.

The source (51544) and destination (80) ports identify the processes/applications that are sending and receiving the data, ensuring that the correct data arrives at the intended process when it gets to the destination computer. Port 80 is commonly used by web servers to listen for Hypertext Transfer Protocol requests. The window size of 2068 indicates that the receiving device will accept data sizes up to 2068 bytes at any one time.

2.4 Statistics

(i) I/O Graph

This I/O Graph shows the rate of packets sent and received per second, we can see that there were 27 packets sent within the 3rd second of the TCP conversation. Furthermore, the graph shows 5 different TCP errors, with two Reset packets received during the 3rd second, a Retransmission and its Acknowledgement in 4th second and a final Acknowledgement in the 5th second for segment that wasn't captured.

(ii) Packet Lengths

Looking at this analysis of packet lengths, we can see there were 38 packets captured in total and that most packets (22, 57.89%) were between 40-79 bytes, with the next most common size between 80-159 (12, 31.58%); 2 UDP packets were between 160-319 bytes and 2 HTTP packets were between 540-1279 bytes brought the overall average packet length to 122.66 bytes. We can see from the Burst Start that the maximum number of packets were sent at 3.173 seconds into the capture, this matches with the peak packets/sec in our IO Graph.